| Course Name: Course Code: | Digital Logic Design Laboratory (DLDL) 316U06L202 | Semester: | II |
| --- | --- | --- | --- |
| Date of Performance: | 11/ 03 /2026 | Batch No: | A1 |
| Student Name: | Arav Arun | Roll No: | 16010125013 |
| Faculty Sign & Date: |  | Grade/Marks: | / 2 |

Experiment No: 8

Title: Implementation of 3-bit Asynchronous Counter using JK Flip-Flops

Aim and Objective of the Experiment:

Design and verify the 3 - Bit Asynchronous Counter using JK flip flops. Design and implement sequential logic circuits and their applications

COs to be achieved:

CO4: Implement sequential logic circuits and applications using MSI devices. CO5: Understand basic principles required for advanced Digital Design

ICs Used: 7476 Asynchronous Counter Design Steps: - Determine the Modulus (MOD) - Decide how many states the counter should have - Example: MOD-8 → counts from 0 to 7 - Choose Type of Flip-Flop - Common choices: JK Flip-Flop or T Flip-Flop - For toggling: - JK → J = K = 1 - T → T = 1 - Connect Flip-Flops in Ripple Fashion - First flip-flop gets external clock - Each next flip-flop gets clock from previous flip-flop’s output (Q) - Set Initial Conditions - Reset all flip-flops to 0 (using clear/reset inputs) - Design for Required MOD (if not power of 2) - Add logic gates (AND/NAND) to reset counter at desired count - Example: MOD-6 → reset when count reaches 6 (110) - Draw the Circuit Diagram - Show: - Flip-flops - Clock connections (ripple) - Reset logic (if any) - Verify Counting Sequence - Check output states (Q0, Q1, Q2…) - Ensure correct counting (binary sequence or truncated sequence) - Logic Diagram: 3-bit Asynchronous counter using J-K Flip-flop Truth Table: Asynchronous Counter Clock State; Outputs. S; Q2; Q1; Q0. 0; 0; 0; 0. 1; 0; 0; 1. 2; 0; 1; 0. 3; 0; 1; 1. 4; 1; 0; 0. 5; 1; 0; 1. 6; 1; 1; 0. 7; 1; 1; 1.

Conclusion: In this experiment, we successfully designed and verified a 3-bit Asynchronous (Ripple) Counter using JK flipflops. We understood how the output of one flip-flop acts as the clock input for the next stage, resulting in a sequential counting operation. The practical implementation helped us observe the ripple effect and timing delay between outputs. Through this experiment, we gained a clear understanding of sequential logic circuits and their applications in digital systems such as counters and frequency division.

Signature of faculty in-charge with Date: